EXP 16 : MATLAB SIMULATION OF SINGLE – BIT ERROR DETECTION AND CORRECTION USING SYNDROME VECTORS IN LINEAR BLOCK CODES

SOFTWARE USED : MATLAB SOFTWARE

CODE :

clc;

clear;

close all;

% Parity-check matrix for a (7,4) linear block code

H = [1 0 1 0 1 0 1;

0 1 1 0 0 1 1;

0 0 0 1 1 1 1];

% Original valid codeword

c = [1 0 1 1 0 0 1];

% Introduce a single-bit error at position 4

r = c;

r(4) = 1 - r(4);

% Calculate syndrome

s = mod(H * r', 2);

% Detect error position

pos = 0;

for i = 1:7

if isequal(H(:,i), s)

pos = i;

break;

end

end

% Correct the error

corrected = r;

if pos > 0

corrected(pos) = 1 - corrected(pos);

end

% Display results in Command Window

disp('SINGLE-BIT ERROR DETECTION AND CORRECTION');

disp('----------------------------------------');

disp(['Original Codeword  : ', num2str(c)]);

disp(['Received Codeword  : ', num2str(r)]);

disp(['Syndrome           : ', num2str(s')]);

if pos > 0

fprintf('Error Position     : %d\n', pos);

disp(['Corrected Codeword : ', num2str(corrected)]);

else

disp('No single-bit error detected.');

end

% Plot the results

figure('Name','Syndrome Error Correction','Color','w');

subplot(3,1,1);

stem(1:7,c,'filled','LineWidth',1.5);

title('Original Codeword');

xlabel('Bit Position');

ylabel('Bit Value');

xlim([0 8]); ylim([-0.2 1.2]);

grid on;

subplot(3,1,2);

stem(1:7,r,'filled','LineWidth',1.5);

title('Received Codeword (Error at Bit 4)');

xlabel('Bit Position');

ylabel('Bit Value');

xlim([0 8]); ylim([-0.2 1.2]);

grid on;

subplot(3,1,3);

stem(1:7,corrected,'filled','LineWidth',1.5);

title('Corrected Codeword');

xlabel('Bit Position');

ylabel('Bit Value');

xlim([0 8]); ylim([-0.2 1.2]);

grid on;

MODEL GRAPH :

RESULT :